IMPORTANT

Cut along the dotted lines and stick each box to your Datamax printer (Either side and on top)

Important instructions for easy reference, your employees will thank you!

## How to Calibrate the

## Datamax Printer:

Press and hold «Play/Pause» button for 10 seconds

Then release when the LED light flashes green.

>> The printer will output 5-10 labels

>> The light LED light will be solid green

## To ensure proper calibration:

- Press and release the «Play/Pause» button 3 times. If it is properly calibrated, only 1 label should come out each time (NOT 1.5 or 0.5 labels).

- Open the printer and roll the labels back in, they should stick out about 5mm.

## You must Re-Calibrate

## The printer if:

- The LED light is NOT solid green.

- The ribbon and/or labels are changed.

- If the label was pulled to hard from the printer in a way that de-calibrates the machine. Be careful when tearing.

- In case of major issues, restart the print «before» calibrating

For the Technician

IP Address: _______________________

Subnet Mask: _____________________

Gatway IP: ________________________